| Experiment Number | 10 |
| --- | --- |
| Date of Experiment | 02/11/2020 |
| Date of Submission | 07/11/2020 |
| Name of the student | Debagnik Kar |
| Roll Number | 1804373 |
| Section | ETC - 06 |

Aim of The Experiment :-

Open Ended 2

Application of Frequency division multiplexing using audio signal

Software Required:-

- MATLAB r2018a

- Audio files (Get it here)

Theory

Multiplexing is a process by which a number of signals can be transmitted through a single channel without interference.

There are mainly two type of multiplexing techniques for analog signals :

- Time division multiplexing

- Frequency division multiplexing.

In this project we will perform application of frequency division multiplexing using 3 audio clips.

Frequency-division multiplexing (FDM) is a technique by which the total bandwidth available in a communication medium is divided into a series of non-overlapping frequency bands, each of which is used to carry a separate signal.

FDM uses a carrier signal at a discrete frequency for each data stream and then combines many modulated signals. When FDM is used to allow multiple users to share a single physical communications medium (i.e. not broadcast through the air), the technology is called frequency-division multiple access (FDMA).

Here  frequency division multiplexing is performed on 3 SSBSC modulated signals (audio clips) with different carrier frequencies. Each modulated signal is allocated with different frequency spectrum created at lets say fc1,fc2,and fc3 and therefore signal m1,m2 and m3 do not interfere with each other.

Code:-

<<<SimpleFDM.m Comment: Generates SSBSC signals from the audio files then transmit it through a noisy in frequency division multiplexed way, upon receiving noise is reduced, demultiplex and then SSBSC demodulated. Then played the message back>>>

%Experiment 10: Open ended 2:

%Application of FDM Multiplexing and Demultiplexing

%Collaboration of

%Debagnik Kar (1804373) and Sayani Ghoroi (1804406)

clear all

clc

close all

%PARAMETERS

bandwidth = 4000; % bandwidth for each frequency band in Hz

media_guard = 300; %

signal_to_noise_ratio = 20;

modudation_ssb = 1;% 1 for Single SSB modulation, 0 for AM

%The first signal will be placed in the third channel, the second in the

%fourth and the third in the fifth.

frec_carrier1 = bandwidth*3;% Carrier frequency in Hz

frec_carrier2 = bandwidth*4;

frec_carrier3 = bandwidth*5;

Fs = frec_carrier3*2+5000;

cutoff_freq_passfilter = 2500;

% 1 show graphics, 0 don't

graphics = 1;

% 1 play sounds, 0 don't

sounds = 1;

%Define filters

[B,A] = butter(4,cutoff_freq_passfilter/(Fs/2));

low_pass = @(S) filter(B,A,S); %function handeling filter() in low_pass variable to know more about it go to

[C1,D1] = butter(2,[bandwidth*2+media_guard bandwidth*3-media_guard]/(Fs/2));

band_filter3 = @(S) filter(C1,D1,S);

[C2,D2] = butter(2,[bandwidth*3+media_guard bandwidth*4-media_guard]/(Fs/2));

band_filter4 = @(S) filter(C2,D2,S);

[C3,D3] = butter(2,[bandwidth*4+media_guard bandwidth*5-media_guard]/(Fs/2));

band_filter5 = @(S) filter(C3,D3,S);

%upload the files

[s1, g1] = audioread('1.wav');

len1 = length(s1);

[s2, g2]= audioread('2.wav');

len2 = length(s2);

[s3, g3]= audioread('3.wav'); % you got rick rolled lol in 2020

len3 = length(s3);

[beep, g4]= audioread('beep-8.wav');

playerbeep = audioplayer(beep,44100);

%are truncated to the length of the minor

min_len = min([len1 len2]);

t = linspace(0,5, min_len);

s1 = s1(1:min_len);

s2 = s2(1:min_len);

s3 = s3(1:min_len);

FLAG = input('STEP 1, the signals are reproduced as they arrive');

%playing sounds

if (sounds > 0)

player = audioplayer(s1,g1);

playblocking(player);

playblocking(playerbeep);

player2 = audioplayer(s2,g2);

playblocking(player2);

playblocking(playerbeep);

player3 = audioplayer(s3,g3);

playblocking(player3);

end

FLAG = input('STEP 2, plot the spectra of the signals as they arrive');

if (graphics > 0)

figure

esps1=abs(fft(s1));

subplot(3,1,1),plot(esps1),grid on,zoom,title('Signal Spectrum 1');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

esps2=abs(fft(s2));

subplot(3,1,2),plot(esps2),grid on,zoom,title('Signal Spectrum 2');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

esps3=abs(fft(s3));

subplot(3,1,3),plot(esps3),grid on,zoom,title('Signal Spectrum 3');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

end;

FLAG = input('STEP 3, Signals are passed through a low pass filter and plotted');

%they go through the low pass filter

s1 = low_pass(s1); %function handling by a variable

s2 = low_pass(s2);

s3 = low_pass(s3);

%Plot

if (graphics > 0)

figure

esps1=abs(fft(s1));

subplot(3,1,1),plot(esps1),grid on,zoom,title('Signal spectrum 1 filtered');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

esps2=abs(fft(s2));

subplot(3,1,2),plot(esps2),grid on,zoom,title('Signal spectrum 2 filtered');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

esps3=abs(fft(s3));

subplot(3,1,3),plot(esps3),grid on,zoom,title('Signal spectrum 3 filtered');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

end

FLAG = input('STEP 4, reproduce the signals after passing them through the filter');

%Played again

if (sounds > 0)

playerbeep = audioplayer(beep,44100);

player = audioplayer(s1,g1);

playblocking(player);

playblocking(playerbeep);

player2 = audioplayer(s2,g2);

playblocking(player2);

playblocking(playerbeep);

player3 = audioplayer(s3,g3);

playblocking(player3);

playblocking(playerbeep);

end

FLAG = input('STEP 5, Signals are modulated to different carriers');

%Modulate

if ( modudation_ssb  > 0)

s1mod = ssbmod(s1,frec_carrier1,Fs);%modulates

s2mod = ssbmod(s2,frec_carrier2,Fs);%modulates

s3mod = ssbmod(s3,frec_carrier3,Fs);%modulates

else

s1mod = ammod(s1,frec_carrier1,Fs);%modulates

s2mod = ammod(s2,frec_carrier2,Fs);%modulates

s3mod = ammod(s3,frec_carrier3,Fs);%modulates

end

%plotted

if (graphics > 0)

figure

esps1=abs(fft(s1mod));

subplot(3,1,1),plot(esps1),grid on,zoom,title('Signal spectrum1 modulated');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

esps2=abs(fft(s2mod));

subplot(3,1,2),plot(esps2),grid on,zoom,title('Signal spectrum2 modulated');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

esps3=abs(fft(s3mod));

subplot(3,1,3),plot(esps3),grid on,zoom,title('Signal spectrum2 modulated');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

end

FLAG = input('STEP 6, The modulated signals are filtered in the determined band and summed');

fs1 = s1mod;

fs2 = s2mod;

fs3 = s3mod;

%added

x = fs1+fs2+fs3;

%plotted again

if (graphics > 0)

figure

esps1=abs(fft(fs1));

subplot(4,1,1),plot(esps1),grid on,zoom,title('Signal spectrum1 modulated and filtered');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

esps2=abs(fft(fs2));

subplot(4,1,2),plot(esps2),grid on,zoom,title('Signal spectrum2 modulated and filtered');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

esps3=abs(fft(fs3));

subplot(4,1,3),plot(esps3),grid on,zoom,title('Signal spectrum3 modulated and filtered');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

espf=abs(fft(x));

subplot(4,1,4),plot(espf),grid on,zoom,title('Summed Spectrum');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

end

FLAG = input('STEP 7, add some noise to the transmitted signal');

if (graphics > 0)

figure

esps1=abs(fft(x));

subplot(2,1,1),plot(esps1),grid on,zoom,title('Full signal spectrum');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

end

x = awgn(x, signal_to_noise_ratio );

if (graphics > 0)

esps1=abs(fft(x));

subplot(2,1,2),plot(esps1),grid on,zoom,title('Full signal spectrum plus some noise'); xlabel("Frequency, Hz");ylabel("Amplitude, dB");

end

FLAG = input('STEP 8, upon arrival each band is filtered');

%signals are received and filtered

demuxs1 = band_filter3(x);

demuxs2 = band_filter4(x);

demuxs3 = band_filter5(x);

%Plotted again

if (graphics > 0)

figure

esps1=abs(fft(demuxs1));

subplot(3,1,1),plot(esps1),grid on,zoom,title('Signal spectrum1 filtered');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

esps2=abs(fft(demuxs2));

subplot(3,1,2),plot(esps2),grid on,zoom,title('Signal spectrum2 filtered');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

esps3=abs(fft(demuxs3));

subplot(3,1,3),plot(esps3),grid on,zoom,title('Signal spectrum3 filtered');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

end

FLAG = input('STEP 9, each recovered band is demodulated to return the signal to the indicated frequency');

%Demodulate

if ( modudation_ssb  > 0)

demods1 = ssbdemod(demuxs1, frec_carrier1,Fs);

demods2 = ssbdemod(demuxs2, frec_carrier2,Fs);

demods3 = ssbdemod(demuxs3, frec_carrier3,Fs);

else

demods1 = amdemod(demuxs1, frec_carrier1,Fs);

demods2 = amdemod(demuxs2, frec_carrier2,Fs);

demods3 = amdemod(demuxs3, frec_carrier3,Fs);

end;

%Plotting

if (graphics > 0)

figure

esps1=abs(fft(demods1));

subplot(3,1,1),plot(esps1),grid on,zoom,title('Demodulated signal1 spectrum');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

esps2=abs(fft(demods2));

subplot(3,1,2),plot(esps2),grid on,zoom,title('Demodulated signal2 spectrum');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

esps3=abs(fft(demods3));

subplot(3,1,3),plot(esps3),grid on,zoom,title('Demodulated signal3 spectrum');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

end

FLAG = input('STEP 10, the recovered signal is passed through a low pass filter');

%played

demods1 = low_pass(demods1);

demods2 = low_pass(demods2);

demods3 = low_pass(demods3);

if (graphics > 0)

figure

esps1=abs(fft(demods1));

subplot(3,1,1),plot(esps1),grid on,zoom,title('Demodulated signal1 spectrum');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

esps2=abs(fft(demods2));

subplot(3,1,2),plot(esps2),grid on,zoom,title('Demodulated signal2 spectrum');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

esps3=abs(fft(demods3));

subplot(3,1,3),plot(esps3),grid on,zoom,title('Demodulated signal3 spectrum');xlabel("Frequency, Hz");ylabel("Amplitude, dB");

end

FLAG = input('STEP 11, Signal reproduce the signal after transmission');

player4 = audioplayer(demods1,g2);

playblocking(player4);

playblocking(playerbeep);

player5 = audioplayer(demods2,g2);

playblocking(player5);

playblocking(playerbeep);

player6 = audioplayer(demods3,g3);

playblocking(player6);

Output/Graph:-

STEP 1, the signals are reproduced as they arrive:

*Three Audios plays

STEP 2, plot the spectra of the signals as they arrive

Fig 10.1: Step 2, spectral plot for the three signals

STEP 3, Signals are passed through a low pass filter and plotted

Fig 10.2: Step 3, Spectral plot for filtered signals

STEP 4, reproduce the signals after passing them through the filter

**Filtered audio signal plays

STEP 5, Signals are modulated to different carriers

Fig 10.3: Step 5, SSBSC Modulated and filtered spectral plot

STEP 6, The modulated signals are filtered in the determined band and summed

Fig 10.4: The three signals are frequency multiplexed

STEP 7, add some noise to the transmitted signal

Fig 10.5: Multiplexed signal spectral plot with and without noise.

STEP 8, upon arrival each band is filtered

Fig 10.6: spectral plot of demultiplexed signals upon receiving.

STEP 9:, each recovered band is demodulated to return the signal to the indicated frequency

Fig 10.6: the signals are SSBSC demodulated

STEP 10, the recovered signal is passed through a low-pass filter

Fig 10.7: spectral plot of the recovered signals

STEP 11, Signal reproduce the signal after transmission

**The received and recovered signals are played

Discussion or Inference of the experiment

From this open-ended project, we learned about the frequency division multiplexing and demultiplexing of signals during communication through a channel and we applied it generate  3 SSBSC modulated message signals (audio clips) with different carriers . we also were able to successfully multiplex ,demultiplex and demodulate the signal with the help of MATLAB to get back the message signals (audio clips)  . Also, all the waveforms were simulated as expected in theory.

Conclusion:-

From this project  we  successfully performed Frequency division multiplexing using audio signal and its demultiplexing using MATLAB and simulated their waveforms.